# Name & Student ID

Name: Daanish Salmaan

Student ID: 20526141

# Title

Initial Project Planning Document – Drawing Robot Software

# Outline of the Problem to be Solved

The objective of the project is to produce software that can program a drawing robot to draw a phrase of text, with a defined font, letter size and overall text area parameters.

The program will recall the font data from the “SingleStrokeFont.txt” file, which defines each ASCII (American Standard Code for Information Interchange) using X and Y co-ordinates and a corresponding pen-up and pen-down state. This data is to be stored in the program to be accessed efficiently while the program runs and the text is drawn.

The software will prompt the user to input a height value for the text. There will be parameters to ensure that the input height is between 4mm and 10mm, outputting an error message if the value Is beyond the desired range. The default height is 18 units, so the code will scale the font accordingly. The code must access a second text file, which contains the text to be drawn by the robot. The code will ensure that the code fits into a width of 100mm, starting a new line of text if the characters do not fit onto the line.

The program will generate G-code commands to control the CNC robot arm, which is processed by an Arduino microcontroller. A RS232 serial port, operating at 115200 baud, will be used for transfer of data between components. Pen Motion (G0, G1), pen control (S0, S1000) and drawing speed (M3, F1000) must also be represented by the generated commands. The program will maintain constant communication with the Arduino chip, allowing for a signal before each command and ending the code, resulting in the pen returning to the start co-ordinates (0,0).

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| fontData[128] | struct FontChar | Stores drawing information for all 128 ASCII characters |
| struct FontChar | struct | Stores co-ordinates for each character |
| textToDraw | char | Stores the desired text to draw from the file |
| textHeight | float | Stores the height inputted by user |
| scaleFactor | float | Used to scale letters (height/18) |
| Xposition, Yposition | float | Tracks pen co-ordinates |
| penState | int | 0 for pen up, 1 for pen down |
| Gcode[100] | char | Builds the gcode |

# Function Declarations

int ReadFontFile(struct FontChar fontData[], char *filename)

Parameters:

fontData – array of structures that store the information for each ASCII character

filename – string containing the name of the font file, in this case SingleStrokeFont.txt

Return value – returns 1 if file found, 0 if there was an error accessing the file

int ReadTextFile(char *buffer, char *filename)

Parameters:

buffer – a character array where the text from the file is stored

filename – string containing the name of the text file,

Return value – returns the number of characters there are in the text file, to be drawn

float GetTextHeight(void)

Return value – returns the height value inputted, but asks for another value if not between 4mm and 10mm

void ScaleFont(struct FontChar *fontData, float scaleFactor)

Parameters:

fontData – pointer to the font data array

scaleFactor – scaling factor (height/18) used to resize the letter co-ordinates

Return value – none

int ReadTextFile(char *buffer, char *filename)

Parameters:

buffer – a character array where the text from the file is stored

filename – string containing the name of the text file,

Return value – returns the number of characters there are in the text file, to be drawn

void GenerateGCode(char *text, struct FontChar *fontData, float scaleFactor)

Parameters:

text – the text string to be drawn

fontData – the array containing font data for all characters

scaleFactor – the scaling factor used for drawing

Return value – none

void GCodeToArduino(char *command)

Parameters:

command – a string containing a single G-Code command to send

Return value – none

void MoveToOrigin(void)

Return value – none

int main(void)

Return value – returns 0 when program runs successfully

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| main() | Full program runs | - | Program completes full process and ends at (0,0) with pen up. |
| GetTextHeight() | Valid input. | 8 entered | Height value accepted |
| GetTextHeight() | Invalid input. | 8 not entered | Error message displayed |
| ReadFontFile() | Load font file | File: SingleStrokeFont.txt | Font data is read and stored correctly in fontData[] |
| ReadTextFile() | Read text | File: TextToDraw.txt | Returns 5, buffer contains "HELLO" |
| ScaleFont() | Apply scaling | input height = 9, scale factor = 0.5 | All X and Y coordinates reduced by half |
| GenerateGCode() | Draw one letter | “H” | Produces correct G-Code sequence for ‘H’ (pen up/down and line moves) |
| GenerateGCode() | Line wrapping | Long sentence exceeding 100 mm width | Starts a new line and applies 5 mm line spacing |
| GCodeToArduino() | Check communication | "G0 X0 Y0" | Arduino confirms message recieved |
| MoveToOrigin() | End sequence | - | Sends S0, G0 X0 Y0; pen up |

# Flowchart(s)

May be included as separate pdf

# AI Statement

I used AI tools to review my work and check whether it met the assignment requirements.
AI was used to identify if I had missed any important functions, data items, or logical flows that should be included in the program design.
The content and structure were written and verified by me, and the AI support was limited to reviewing and suggesting improvements to ensure completeness and clarity.